……..........................................................

Dr. Pongkarn Chakthranont

โครงการ P2451496 งบยุทธ_CCU68_การต่อยอดและพัฒนาเทคโนโลยี Carbon capture and utilization (CCU) ที่มีความต้องการจากภาคอุตสาหกรรม และขับเคลื่อน National CCUS Alliance ร่วมกับเครือข่ายพันธมิตร

Sothearoth Heng

Under the supervision of

Dr. Pongkarn Chakthranont

ใบสั่งจ้างเลขที่ PO25005060

จ้างเหมาพัฒนางานจ้างพัฒนาการขยายขนาดเซลล์ไฟฟ้าเพื่อการผลิตแก๊สสังเคราะห์จาก CO2 เชิงเคมีไฟฟ้า

งวดที่ 6: 1 September 2568 – 30 September2568

September monthly report

- To-do list in September

- Mitigation of water accumulation: 100cm2 electrolyzer – CO2 electrolysis of 0.5Ni-SAC under a pure CO2 at 100mA/cm2: Increase CO2 flow rate (500-1000 ml/min)

- Effect of Flow field configuration: Parallel vs Quad-serpentine

- Experimental Procedure

Synthesis of NiSAC by pyrolysis method

The synthesis of NiSAC catalyst was followed by our previous work. Initially, nickel phthalocyanine (NiPc), urea, and dicyanamide precursors were stoichiometrically weighed according to mass ratios of x:5:5, wherein urea and dicyanamide were maintained at 5 g each while varying the NiPc content (x = 0.25, 0.5, 1.0, and 2.0 g). Subsequently, the mixture was thoroughly mixed and ground until a homogeneous midnight blue powder was obtained. The fine powder was then transferred into a quartz boat covered with a capsule lid and subjected to polymerization at 800 °C (ramp rate = 10 °C·min⁻¹) in a horizontal tube furnace under an argon atmosphere (flow rate = 200 sccm) for 2 h. The as-prepared xNiSAC catalysts with different NiPc contents were finally obtained and labeled as 0.25NiSAC, 0.5NiSAC, 1.0NiSAC, and 2.0NiSAC.

Fabrication of IrO2/TiPt felt anode electrode

The IrO2/TiPt-felt anode electrode was prepared using a thermal deposition method involving multiple sequential steps. Initially, the TiPt felt (Purchased from ...) substrate was ultrasonically cleaned with deionized water and acetone to remove any surface contaminants. The cleaned substrate was subsequently etched in a 6M HCl solution at 90°C for 45 min to enhance surface roughness and adhesion properties. The dip-coating solution was prepared by dissolving 200 mg of IrCl6·xH2O in 18 mL of isopropanol (IPA) containing 10% v/v HCl (2 mL of conc. HCl). The etched TiPt felt substrate was then immersed in the dip-coating solution, followed by drying at 100°C for 5 min in an oven and subsequent calcination at 500°C for 10 min in a muffle furnace. After cooling to room temperature, the treated substrate was reweighed to determine mass gain. This dipping-drying-calcination cycle was repeated several times until the desired IrO2 mass loading (10.2 mg/cm2) was achieved.

Preparation of NiSAC cathode electrode via spray coating method

The NiSAC gas-diffusion electrode was fabricated by spray coating onto Sigracet 39BB carbon paper or hydrophilic carbon cloth substrates. Typically, 150 mg of as-prepared NiSAC and 381 μL of XC-2 ionomer were ultrasonically dispersed in 10 mL of isopropanol (IPA) for at least 1 h to form a homogeneous dark colloidal suspension. The ink suspension was subsequently spray-coated onto a 6x6 cm2 carbon substrate using an automated, custom-built spraying setup equipped with a syringe pump operating at a flow rate of 0.6 mL min⁻1 on a hotplate maintained at 110°C, yielding a catalyst mass loading of 1.8 ± 0.1 mg.cm⁻2.

Electrochemical measurements in membrane-electrode assembly (MEA) electrolyzer

100cm2 MEA electrolyzer operational set-up

A 100 cm2 MEA electrolyzer was used to evaluate the CO2RR performance under modulated operating conditions. The Ni foam or IrO2-TiPt felt (10.5 x 10.5 cm2), bipolar membrane (BPM, 15.6 x 16.8 cm2), and 0.5Ni-SAC (10.5 x 10.5 cm2) were employed as the anode, membrane, and cathode electrode, respectively. The MEA was assembled with 0.25µm PTFE gasket and tightened at a compression force of 25 Nm. 1.0 L of 0.25 M KOH was recirculated and used throughout the experiment as the anolyte at 100 ml/min. The humidified CO2 gas stream was introduced into the cathodic inlet with a total CO2 flow rate of 200-1000 ml/min. Prior the test, the electrochemical impedance spectroscopy (EIS) and Cyclic voltammetry were measured. Meanwhile, the cathodic outlet was directly injected into an online gas-chromatography for gas products analysis.

- Results

Mitigation of water accumulation by increasing cathodic inlet gas stream

According to our preliminary results, the water accumulation (liquid droplets) at the flow-field channel of the cathode side is the main issue that occurs with a parallel flow field design because the pressure drop at the exit is too low to force out the water, causing the flow field channels to become clogged and the reactant gases to be distributed unevenly, decreasing overall performance. We therefore increase the CO2 flow rate to help improve accumulated water removal in the BPM system. The CO2 electrolysis of the BPM system was evaluated under 1000ml/min of 50%CO2/50N2 cathodic gas stream and 0.25M KOH at 100 ml/min at 100 mA/cm2 with a cell configuration of 0.5NiSAC (1.8 mg/cm2) |BPM| IrO2-TiPt. The MEA electrolyzer was assembled with key components including a 0.3 µm PTFE gasket (10.5 × 10.5 cm2), a BPM size of 15.6 × 16.8 cm2., and a compression force of 120 in·lbs. As shown in Fig. 1, Increasing total cathodic flow rate by mixing 500ml/minCO2 and 500ml/minN2 resulted in abruptly inclined in cell voltage which might result from CO2 starvation at the catalyst surface at high applied current density. Interestingly, once a pure 500ml/minCO2 was replaced, the overall system still worked properly.

Figure 1. CO2 electrolysis of 0.5Ni-SAC in a 100cm2 MEA electrolyzer under 1000ml/min of 50%CO2/50%N2, 100ml/min 0.25M KOH, and IrO2-TiPt anode at 100mA/cm2

Based on the above-mentioned result, we encountered that increasing the total flow rate by using the diluted 50%CO2 condition resulted in an immediate failure, thus a pure 1000ml/min CO2 should be tried. Interestingly, as depicted in Fig. 2, further increasing the CO2 flow rate to 1000ml/min enables to sustain a high FE(CO) > 90% for up to 13h before cell failure. Besides the improvement of system stability, the water accumulation was still observed at the GDE backside and flow-field channel which could be attributed to the gradual cell voltage rise over time and somehow caused a sudden overshoot above 4.0V, triggering HER activity.

Figure 2. CO2 electrolysis of 0.5Ni-SAC in a 100cm2 MEA electrolyzer under 1000ml/min of pure CO2, 100ml/min 0.25M KOH, and IrO2-TiPt at 100mA/cm2

Effect of Flow field configuration: Parallel vs Quad-serpentine

To ascertain the effect of flow-field channel configuration, the cathodic flow-field channel was replaced by serpentine flow-field channel, while still using the parallel flow-field channel for the anode side. The electrochemical reduction of CO2 was experimentally conducted using a BPM system deployed 0.5NiSAC as a cathode under a humidified pure 200sccm CO2 with 2L of 0.25M KOH and IrO2 anode at 100mA/cm2. The BPM system with 0.5Ni-SAC initially achieved a maximum FE(CO) >95% at a full-cell voltage at 3.3V, and sustaining the FE(CO) >85% over 11h of operational duration. This result indicated the importance of the flow-field channel configuration for potentially removing accumulated droplets at the cathodic flow-field. However, the accumulated droplets are still observed in the serpentine flow-field which could be attributed to mass transport limitation over time, causing a marginal increase in over-potential and HER activity.

Figure 3. (a) Comparison of parallel and serpentine flow fields for CO2 electrolysis of 0.5Ni-SAC in a 100cm2 MEA electrolyzer under 200ml/min of CO2, 100ml/min 0.25M KOH, and IrO2-TiPt anode at 100mA/cm2 and (b) under 200 and 1000 ml/min CO2 flow rate, respectively.

- Upcoming tasks

  - 100cm2 MEA electrolyzer – CO2 electrolysis of serpentine-BPM system under a dried 200ml/min CO2, 100ml/min of 0.25M KOH, and IrO2-TiPt anode at 100mA/cm2

  - 100cm2 MEA electrolyzer – CO2 electrolysis of serpentine-BPM system under a humidified 500ml/min CO2, 100ml/min of 0.25M KOH, and IrO2-TiPt anode at 100mA/cm2